# Supplementary Information

# S1. Sources of Error and Limitations

Script generation in this study used ChatGPT (GPT-5). During iterative development, four geometry issues appeared: incorrect junction alignment, solid instead of hollow channels, twisted hollow tubes, and unstable SVG/DXF output. Each problem arose when prompts lacked sufficient specificity—consistent with prior observations—and was resolved through prompt refinement and automatic geometry checks embedded in the script.

Consistency within a single environment: Prompts were tested multiple times in the same GPT environment. Repeated execution of a finalized script consistently produced identical STL files and matching MD5 hashes, confirming deterministic behavior at the execution stage.

Variability across environments: Running identical prompts in a different GPT environment (or under a different user account) produced code with minor variations, resulting in different STL MD5 hashes after geometry export. Additional tests using locked prompt structures and fixed generation settings reduced but did not eliminate this variability. The results confirm that differences originate in the probabilistic code-generation stage, not in the deterministic execution of the finalized workflow.

To address this, the finalized workflow was compressed into a single master prompt (S5). This reduced inter-script variability across environments, though complete code-level identity was not achieved.

Practical guidance for users:

Unclear or incomplete instructions can produce geometric errors; precise parameter specification is essential.

The master prompt in S5 is recommended for better cross-environment consistency.

Each exported STL file should be validated whenever parameters or device layouts change.

Because all printing parameters and materials were held constant throughout this study, observed errors were repeatable—a useful property for future closed-loop geometric compensation strategies.

# S2. Unsuccessful Prompts (Phase 1)

Model and access information. All prompts in this section (Prompts 1–10) were executed using ChatGPT (GPT-5) via the ChatGPT web interface. LLM outputs are inherently probabilistic and may shift across model updates, so responses can vary by user, access date, and environment. Reproducibility in this study reflects the deterministic execution of the finalized Python workflow, not the invariance of the initial LLM-generated code. The master prompt in S5 is recommended for improved cross-user reproducibility.

The following prompts (1–10) represent the initial, unsuccessful attempts to generate the code.

## Prompt 1 - Baseline geometry generation

Write a Python script to generate simple microfluidic channel centrelines for a spiral, a T-junction, and a Y-junction using parametric equations. Export the centrelines as STL-ready paths.

## Prompt 2 - Hollow circular channels

Modify the script so that each centreline is swept with a circular profile to form hollow microfluidic channels with a specified outer diameter and wall thickness.

## Prompt 3 - Watertight STL requirement

Ensure that the generated STL files are watertight and suitable for slicing. Avoid relying solely on slicer auto-repair.

## Prompt 4 - Robust junction geometry

The T-junction and Y-junction STLs show non-manifold errors at the junctions. Redesign the geometry so junctions are robust and manifold, even after Boolean operations.

## Prompt 5 - Precise T and Y Topology

Fix the T-junction so it is a true T (one horizontal main channel and one vertical branch from the centre). Fix the Y-junction so it has two symmetric branches at a specified angle (e.g., 45°).

## Prompt 6 - Multi-format export and annotations

Extend the script to export: STL for fabrication, DXF with correct line widths, SVG images with clear annotations and ensure that each channel segment remains a separate polyline in DXF/SVG.

## Prompt 7 - Logging and reproducibility

Add logging to the script: CSV file recording device type, generation time, and STL hash, Prompt log file that stores the text prompt used for SVG generation. This is for experimental reproducibility.

## Prompt 8 - API-assisted SVG generation with fallback

Integrate the OpenAI API to generate SVG drawings of the microfluidic layouts when an API key is available. Include a timeout and a local SVG fallback if the API is unavailable.

## Prompt 9 - Basic Description

Write a Python script for design and fabrication of hollow microfluidic devices for a spiral, a T-junction, and a Y-junction device. In this script, Python is used for geometry creation and fabrication ready STL export, and the API is used for SVG image generation. Using width=500µm, wall ratio=0.30, Spiral: ro=200µm, s=2000µm, loops=5. T and Y-Junction=2000 µm (main length), v=1500µm (branch length), angle =45 degrees for Y-junction.

## Prompt 10 - Final prompt that reproduces the full code

Write a single, complete Python script that generates Spiral, T-junction, and Y-junction microfluidic devices, uses circular hollow channels with parameters channel width w = 500 µm, wall ratio 0.30, spiral parameters r0 = 200 µm, s = 2000 µm, loops = 5, T/Y parameters h = 2000 µm, v = 1500 µm, Y angle = 45°. Builds hollow channels using a robust CAD strategy: outer solid sweep − inner solid sweep (OpenSCAD Boolean difference). Produces watertight STL files suitable for slicing without auto-repair. Exports DXF and SVG drawings where each channel is a separate polyline. Uses GPT-5 via the OpenAI API to generate SVGs when an API key is available, with a local SVG fallback. Adds clear geometry annotations to DXF/SVG outputs. Logs device runs to a CSV file and stores SVG prompts in a prompt log. Includes OpenSCAD and trimesh fallbacks, robust error handling, and reproducibility metadata. Output directory should be created automatically, and the script should run end-to-end when executed.

# S3. Successful Iterative Prompt Conversation (Phase 2)

Model and access information. All prompts in this section (Prompts 1–9) were executed using ChatGPT (GPT-5) via the ChatGPT web interface. As in S2, reproducibility reflects deterministic execution of the finalized workflow. The master prompt in S5 is recommended for improved cross-user reproducibility.

The following conversation log documents the successful chain-of-thought strategy used to generate the final code.

## 1. Conversation Turn 1 [Model: ChatGPT GPT-5]

- User: I'm using an FDM printer with PLA and a 0.2 mm nozzle. I want to build a single Python script that generates a spiral, a T-junction, and a Y-junction microfluidic device. It should export STL for printing and DXF + SVG for verification. I also want reproducibility. How should we approach this from scratch?

- ChatGPT: Let’s treat this like a small “generator pipeline” with one source of truth. Recommendation (and why): Define each device as a parametric centerline (spiral curve, T lines, Y lines). This makes DXF/SVG straightforward and keeps everything consistent. Generate STL in a way slicers like watertight solids, not surface-only shells. For reproducibility: keep fixed sampling counts (no adaptive randomness) and do a stable STL canonicalization step before hashing. Printers note (0.2 mm nozzle): It’s good for smaller features, but sensitive to thin walls and messy meshes. So we’ll avoid fragile wall geometry and keep booleans controlled. Next, we should lock: (1) geometry strategy per device, (2) parameter units, (3) outputs and logging.

## 2. Conversation Turn 2 [Model: ChatGPT GPT-5]

- User: Let’s lock the geometry strategy per device (spiral vs junctions), units, and outputs before writing any code.

- ChatGPT: Here’s what I recommend locking: Geometry strategy Spiral: use a direct hollow tube volume sweep (watertight wall volume). Reason: spirals don’t have intersection junctions; booleans can introduce non-manifold artifacts on tight curves. T and Y junctions: use outer solid minus inner solid boolean difference via OpenSCAD. Reason: junction intersections are where sweep unions are most likely to become non-manifold; OpenSCAD difference is reliable. Units Keep user parameters in micrometers (µm) for readability, convert to mm internally right before geometry. Outputs STL for printing DXF + SVG for verification (centerlines as separate polylines) CSV log: session id, device name, runtime, file paths, hash Next, let’s pick a default parameter set that is realistic for FDM + 0.2 mm nozzle.

## 3. Conversation Turn 3 [Model: ChatGPT GPT-5]

- User: Recommend default parameters in µm for w, internal d, r0, s, loops, h, v, angle, and explain why they’re reasonable for 0.2 mm nozzle FDM.

- ChatGPT: Here’s a solid starting set (in µm) that balances “microfluidic-like” scale with what a 0.2 mm nozzle can produce: w = 600: small outer size but not too tiny. internal_d = 400: lumen that’s not extremely small, so it’s less likely to close from extrusion. r0 = 200: compact spiral start. s = 2100: generous spacing so turns don’t fuse. loops = 5: enough spiral length for testing. h = 1900, v = 1500: compact T/Y coupon dimensions. angle = 40°: moderate Y split. Why this works with a 0.2 mm nozzle: 0.2 mm nozzles often print line widths ~0.22–0.28 mm. This scale is small but still printable if the wall is made slicer-friendly (next step). The spiral spacing avoids near-touching paths that can turn into fused walls. Next, we lock a practical compensation method to make walls more printable without shrinking the lumen.

## 4. Conversation Turn 4 [Model: ChatGPT GPT-5]

- User: Suggest a simple compensation strategy that keeps internal_d fixed but makes walls more printable and pick a fixed value.

- ChatGPT: Best practical strategy: outward thickening. Keep internal_d (the lumen) unchanged. Increase only the outer diameter by a fixed amount outward. I recommend locking: OUTWARD_THICKEN_MM = 0.2. Why 0.2 mm: It’s a big enough boost that walls stop landing right on the slicer’s minimum-perimeter edge. It improves reliability without changing your intended lumen size. Next, we lock file/folder layout, filenames, and logs.

## 5. Conversation Turn 5 [Model: ChatGPT GPT-5]

- User: Lock OUTDIR, CSV and prompt log paths, and fixed base filenames for spiral/tjunction/yjunction outputs.

- ChatGPT: Locked recommendations: OUTDIR = "fdm_outward_fixed300_output" CSV_PATH = OUTDIR/device_runs.csv PROMPT_LOG = OUTDIR/prompt_log.txt Fixed base names (each produces STL/DXF/SVG): spiral_outward15 tjunction_outward15 yjunction_outward15 CSV should record: session id, device, runtime, STL path, DXF path, SVG path, STL hash. Next, we lock the OpenSCAD path and the exact calling pattern, so boolean output is repeatable.

## 6. Conversation Turn 6 [Model: ChatGPT GPT-5]

- User: Lock OPENSCAD_EXE path and specify the exact OpenSCAD command pattern for boolean difference (outer minus inner).

- ChatGPT: OPENSCAD_EXE = r"C:\Program Files\OpenSCAD\openscad.exe" Boolean difference call pattern: Write a temporary .scad containing difference(){ union(){ import(outer...); } union(){ import(inner...); } } Run OpenSCAD as: [OPENSCAD_EXE, "--render", "-o", outpath_abs, scad_path] That’s reliable across runs and avoids fragile in-Python boolean engines. Next, we lock the centerline definitions for Spiral/T/Y and the “true T” constraint (only upward branch).

## 7. Conversation Turn 7 [Model: ChatGPT GPT-5]

- User: Lock the exact centerline definitions for spiral, T, and Y (including ‘true T’: upward branch only), and also lock fixed sampling counts and segments.

- ChatGPT: Locked: Spiral centerline (µm) th = linspace(0, 2piloops, 1200) r = r0 + sth/(2pi) x = rcos(th), y = rsin(th), z = 0 T-junction centerlines (µm) — true T Main: [-h/2, 0, 0] → [h/2, 0, 0] Branch: [0, 0, 0] → [0, v, 0] (upward only) Y-junction centerlines (µm) Main: [-h/2, 0, 0] → [0, 0, 0] Branches: to (vcos(angle), ±vsin(angle), 0) Fixed geometry resolution: Spiral samples: 1200 Tube segments: 64 Next, we lock the reproducibility mechanics: canonicalization + hashing rules.

## 8. Conversation Turn 8 [Model: ChatGPT GPT-5]

- User: Lock how STL hashing will be done, when canonicalize_stl_inplace is applied, and what goes into the CSV row.

- ChatGPT: Locked: STL hash = MD5 of the output STL file (after any canonicalization step). Apply canonicalize_stl_inplace() after OpenSCAD booleans for T and Y (booleans often reorder faces). Spiral can be hashed directly (it’s generated deterministically), but it can also be canonicalized if you want maximum stability. CSV row fields (one row per device): session id device name runtime seconds stl path dxf path svg path stl_md5 Next, we lock the exact “signature” print lines and then we can generate the full code.

## 9. Conversation Turn 9 [Model: ChatGPT GPT-5]

- User: Lock the exact print lines to include in main() and in each builder, then generate the full script as one file and output only code.

- ChatGPT: Code.

10. Conversation Turn 10 [Model: ChatGPT GPT-5]

- User: Keep everything same and add a compensation step where the measured dimensions are reinserted into the python interface for each geometry and it carries out automatic correction/compensation and generates new stl file for the geometries using exact same generation logic as above just compensation applied and generate python code.

- ChatGPT: Code.

# S4. Final Generated Python Script

The following Python script (CAD_outward15_Tfix_compensated.py) is the direct output generated from the successful iterative prompt conversation described in Section S3. This script was used to generate the experimental samples presented in the main text.

(Note: The code is included below in its entirety to ensure reproducibility independent of external repository availability.)

# CAD_outward15_Tfix_compensated.py

# Measurement-based compensation loop tailored to adjust w and h independently per geometry.

# Each geometry maintains its own independent design state until it reaches tolerance.

import os, math, time, uuid, hashlib, tempfile, subprocess, csv, struct

import numpy as np

import trimesh, svgwrite, ezdxf

from types import SimpleNamespace

OUTDIR = "fdm_outward_fixed300_output"

CSV_PATH = os.path.join(OUTDIR, "device_runs.csv")

PROMPT_LOG = os.path.join(OUTDIR, "prompt_log.txt")

os.makedirs(OUTDIR, exist_ok=True)

OPENSCAD_EXE = r"C:\Program Files\OpenSCAD\openscad.exe"

OUTWARD_THICKEN_MM = 0.2

HOLLOW_BOOL_EPS_MM = 0.15

TUBE_SEGMENTS = 64

SPIRAL_SAMPLES = 1200

COMP_TOLERANCE_UM = 5.0

DEFAULT = {"w": 600, "internal_d": 400, "r0": 200, "s": 2100, "loops": 5,

"h": 1900, "v": 1500, "angle": 40}

def log_prompt(kind, text):

with open(PROMPT_LOG, "a", encoding="utf-8") as f:

f.write(f"\n{'-'*60}\n{time.strftime('%Y-%m-%d %H:%M:%S')} | {kind}\n{text}\n")

def um2mm(x):

return np.asarray(x, float) / 1000.0

def safe_norm(v):

n = np.linalg.norm(v)

return v / n if n > 1e-12 else v

def md5_of_file(path):

if not os.path.exists(path):

return "N/A"

h = hashlib.md5()

with open(path, "rb") as f:

for c in iter(lambda: f.read(4096), b""):

h.update(c)

return h.hexdigest()

def canonical_md5(stl_path):

return md5_of_file(stl_path)

def canonicalize_stl_inplace(stl_path, decimals=6):

if not os.path.exists(stl_path):

return

m = trimesh.load(stl_path, force="mesh")

if isinstance(m, trimesh.Scene):

m = trimesh.util.concatenate(tuple(m.geometry.values()))

if not isinstance(m, trimesh.Trimesh) or m.faces is None or len(m.faces) == 0:

return

v = np.asarray(m.vertices, dtype=np.float64)

f = np.asarray(m.faces, dtype=np.int64)

v = np.round(v, decimals=decimals)

tris = v[f]

keys = np.round(tris.reshape(len(tris), -1), decimals=decimals)

order = np.lexsort(keys.T[::-1])

tris = tris[order]

e1 = tris[:, 1] - tris[:, 0]

e2 = tris[:, 2] - tris[:, 0]

n = np.cross(e1, e2)

nn = np.linalg.norm(n, axis=1)

nn[nn == 0] = 1.0

n = (n / nn[:, None]).astype(np.float32)

header = b"CANONICAL_STL_MD5_STABLE".ljust(80, b" ")

with open(stl_path, "wb") as fp:

fp.write(header[:80])

fp.write(struct.pack("<I", tris.shape[0]))

for i in range(tris.shape[0]):

fp.write(struct.pack("<3f", *n[i]))

fp.write(struct.pack("<3f", *tris[i, 0].astype(np.float32)))

fp.write(struct.pack("<3f", *tris[i, 1].astype(np.float32)))

fp.write(struct.pack("<3f", *tris[i, 2].astype(np.float32)))

fp.write(struct.pack("<H", 0))

def parallel_transport(P):

P = np.asarray(P, float)

N = len(P)

T, Nn, Bb = np.zeros((N, 3)), np.zeros((N, 3)), np.zeros((N, 3))

for i in range(N):

if i == 0:

T[i] = safe_norm(P[1] - P[0])

elif i == N - 1:

T[i] = safe_norm(P[i] - P[i - 1])

else:

T[i] = safe_norm(P[i + 1] - P[i - 1])

up = np.array([0, 0, 1.])

if abs(np.dot(up, T[0])) > 0.95:

up = np.array([1., 0, 0])

Nn[0] = safe_norm(np.cross(T[0], np.cross(up, T[0])))

Bb[0] = safe_norm(np.cross(T[0], Nn[0]))

for i in range(1, N):

v = safe_norm(np.cross(T[i - 1], T[i]))

s = np.linalg.norm(np.cross(T[i - 1], T[i]))

c = float(np.clip(np.dot(T[i - 1], T[i]), -1., 1.))

if s < 1e-6:

Nn[i] = Nn[i - 1]

Bb[i] = safe_norm(np.cross(T[i], Nn[i]))

continue

theta = math.atan2(s, c)

K = np.array([[0, -v[2], v[1]], [v[2], 0, -v[0]], [-v[1], v[0], 0]])

R = np.eye(3) + math.sin(theta) * K + (1 - math.cos(theta)) * (K @ K)

Nn[i] = safe_norm(R @ Nn[i - 1])

Bb[i] = safe_norm(np.cross(T[i], Nn[i]))

return T, Nn, Bb

def sweep_hollow_tube_volume(points_mm, outer_d_mm, inner_d_mm, segments=64):

P = np.asarray(points_mm, float)

if len(P) < 2:

raise ValueError("Need at least 2 points to sweep a tube.")

R = max(float(outer_d_mm) / 2.0, 1e-6)

r = max(float(inner_d_mm) / 2.0, 1e-6)

if r >= R:

r = 0.9 * R

T, Nn, Bb = parallel_transport(P)

th = np.linspace(0, 2 * np.pi, segments, endpoint=False)

ct, st = np.cos(th), np.sin(th)

Vout, Vin = [], []

for i in range(len(P)):

n, b, c = Nn[i], Bb[i], P[i]

circle = (n[None, :] * ct[:, None] + b[None, :] * st[:, None])

Vout.append(c + circle * R)

Vin.append(c + circle * r)

Vout = np.vstack(Vout)

Vin = np.vstack(Vin)

def ring_side_faces(off, rings, segs, flip=False):

F = []

for i in range(rings - 1):

a0 = off + i * segs

a1 = off + (i + 1) * segs

for j in range(segs):

a = a0 + j; b = a0 + (j + 1) % segs

c = a1 + j; d = a1 + (j + 1) % segs

F += [[a, c, b], [b, c, d]]

F = np.asarray(F, int)

return F[:, ::-1] if flip else F

rings = len(P)

segs = segments

Fout = ring_side_faces(0, rings, segs, flip=False)

Fin = ring_side_faces(len(Vout), rings, segs, flip=True)

V = np.vstack([Vout, Vin])

F_list = [Fout, Fin]

start_out0 = 0

start_in0 = len(Vout)

Fs = []

for j in range(segs):

o0 = start_out0 + j; o1 = start_out0 + (j + 1) % segs

i0 = start_in0 + j;  i1 = start_in0 + (j + 1) % segs

Fs += [[o0, o1, i0], [i0, o1, i1]]

end_out0 = (rings - 1) * segs

end_in0 = len(Vout) + (rings - 1) * segs

for j in range(segs):

o0 = end_out0 + j; o1 = end_out0 + (j + 1) % segs

i0 = end_in0 + j;  i1 = end_in0 + (j + 1) % segs

Fs += [[o0, i0, o1], [i0, i1, o1]]

F_list.append(np.asarray(Fs, dtype=int))

F = np.vstack(F_list)

m = trimesh.Trimesh(vertices=V, faces=F, process=True)

m.fix_normals()

if not m.is_watertight:

m.fill_holes()

m.remove_duplicate_faces()

m.remove_unreferenced_vertices()

return m

def extend_polyline_at_start(P, extension_mm):

P = np.asarray(P, float).copy()

if len(P) < 2:

return P

d = safe_norm(P[0] - P[1])

P[0] = P[0] + d * extension_mm

return P

def extend_polyline_at_end(P, extension_mm):

P = np.asarray(P, float).copy()

if len(P) < 2:

return P

d = safe_norm(P[-1] - P[-2])

P[-1] = P[-1] + d * extension_mm

return P

def sweep_solid_tube(points_mm, radius_mm, segments=64, cap_start=True, cap_end=True):

P = np.asarray(points_mm, float)

if len(P) < 2:

raise ValueError("Need at least 2 points to sweep a tube.")

radius_mm = max(float(radius_mm), 1e-6)

T, Nn, Bb = parallel_transport(P)

th = np.linspace(0, 2 * np.pi, segments, endpoint=False)

ct, st = np.cos(th), np.sin(th)

rings = []

for i in range(len(P)):

n, b, c = Nn[i], Bb[i], P[i]

rings.append(c + (n[None, :] * ct[:, None] + b[None, :] * st[:, None]) * radius_mm)

V = np.vstack(rings)

def side_faces(rings_count, segs):

F = []

for i in range(rings_count - 1):

a0 = i * segs; a1 = (i + 1) * segs

for j in range(segs):

a = a0 + j; b = a0 + (j + 1) % segs

c = a1 + j; d = a1 + (j + 1) % segs

F += [[a, c, b], [b, c, d]]

return np.asarray(F, int)

F_list = [side_faces(len(P), segments)]

if cap_start:

center_idx = len(V)

V = np.vstack([V, P[0][None, :]])

start0 = 0

for j in range(segments):

a = start0 + j; b = start0 + (j + 1) % segments

F_list.append(np.array([[center_idx, b, a]], dtype=int))

if cap_end:

center_idx = len(V)

V = np.vstack([V, P[-1][None, :]])

end0 = (len(P) - 1) * segments

for j in range(segments):

a = end0 + j; b = end0 + (j + 1) % segments

F_list.append(np.array([[center_idx, a, b]], dtype=int))

F = np.vstack(F_list)

m = trimesh.Trimesh(vertices=V, faces=F, process=True)

m.fix_normals()

if not m.is_watertight:

raise ValueError("Solid sweep unexpectedly not watertight. Check path degeneracy.")

return m

def export_difference(outer_meshes, inner_meshes, outpath):

if not (OPENSCAD_EXE and os.path.exists(OPENSCAD_EXE)):

raise RuntimeError("OpenSCAD not found. Set OPENSCAD_EXE correctly.")

outpath_abs = os.path.abspath(outpath)

os.makedirs(os.path.dirname(outpath_abs), exist_ok=True)

tmp = tempfile.mkdtemp(prefix="scad_diff_")

outer_stls, inner_stls = [], []

for i, m in enumerate(outer_meshes):

p = os.path.join(tmp, f"outer{i}.stl"); m.export(p)

outer_stls.append(os.path.abspath(p).replace('\\', '/'))

for i, m in enumerate(inner_meshes):

p = os.path.join(tmp, f"inner{i}.stl"); m.export(p)

inner_stls.append(os.path.abspath(p).replace('\\', '/'))

scad = os.path.join(tmp, "diff.scad")

with open(scad, "w", encoding="utf-8") as f:

f.write("difference(){\n")

f.write("  union(){\n")

for p in outer_stls:

f.write(f'    import("{p}");\n')

f.write("  }\n")

f.write("  union(){\n")

for p in inner_stls:

f.write(f'    import("{p}");\n')

f.write("  }\n")

f.write("}\n")

cmd = [OPENSCAD_EXE, "--render", "-o", outpath_abs, scad]

proc = subprocess.run(cmd, stdout=subprocess.PIPE, stderr=subprocess.PIPE, text=True)

if proc.returncode != 0:

raise RuntimeError(f"OpenSCAD difference failed.\n{proc.stderr}")

print(f"✅ OpenSCAD difference → {outpath_abs}")

def outward_thicken_params(outer_d_mm, wall_mm, delta_mm):

return outer_d_mm + 2 * delta_mm, wall_mm + delta_mm

def annotation_text(kind, p, iteration=0):

extra = ""

if kind == "Spiral":

extra = f" | loops={p.loops} r0={p.r0}µm s={p.s}µm"

elif kind == "T-junction":

extra = f" | h={p.h}µm v={p.v}µm"

elif kind == "Y-junction":

extra = f" | h={p.h}µm v={p.v}µm θ={p.angle}°"

tag = "" if iteration == 0 else f" [COMP iter={iteration}]"

return f"{kind} | w={p.w}µm | internal_d={p.internal_d}µm | outer+0.2mm{extra}{tag}"

def export_dxf_multi(kind, paths_um, p, base, iteration=0):

doc = ezdxf.new(setup=True); msp = doc.modelspace()

outer_mm = p.w / 1000.0

outer2_mm, _ = outward_thicken_params(outer_mm, 0.0, OUTWARD_THICKEN_MM)

outer2_um = outer2_mm * 1000.0

all_pts = []

for path in paths_um:

c2 = np.asarray(path, float)[:, :2]

msp.add_lwpolyline(c2, dxfattribs={"const_width": outer2_um / 2.0, "layer": "0"})

all_pts.append(c2)

all_pts = np.vstack(all_pts)

xmin, xmax = float(np.min(all_pts[:, 0])), float(np.max(all_pts[:, 0]))

ymin, ymax = float(np.min(all_pts[:, 1])), float(np.max(all_pts[:, 1]))

W, H = xmax - xmin, ymax - ymin

cx, cy = (xmin + xmax) / 2.0, ymax + 0.25 * H

text_height = max(35.0, 0.056 * H)

text_str = annotation_text(kind, p, iteration)

t1 = msp.add_text(text_str, dxfattribs={"height": text_height, "layer": "0"})

t2 = msp.add_text(text_str, dxfattribs={"height": text_height, "layer": "0"})

try:

t1.set_placement((cx, cy)); t2.set_placement((cx + 0.02 * text_height, cy))

except Exception:

t1.dxf.insert = (cx, cy); t2.dxf.insert = (cx + 0.02 * text_height, cy)

out = f"{base}.dxf"; doc.saveas(out)

print(f"📐 DXF → {out}"); return out

def export_svg_local_multi(kind, paths_um, p, base, iteration=0):

all_pts = np.vstack([np.asarray(path, float)[:, :2] for path in paths_um])

xmin, xmax = float(np.min(all_pts[:, 0])), float(np.max(all_pts[:, 0]))

ymin, ymax = float(np.min(all_pts[:, 1])), float(np.max(all_pts[:, 1]))

W, H = xmax - xmin, ymax - ymin

margin = 0.45 * max(W, H)

vb_minx, vb_miny = xmin - margin, ymin - margin

vb_width, vb_height = W + 2 * margin, H + 2 * margin

outer_mm = p.w / 1000.0

outer2_mm, _ = outward_thicken_params(outer_mm, 0.0, OUTWARD_THICKEN_MM)

stroke_width_um = (outer2_mm * 1000.0) / 3.0

dwg = svgwrite.Drawing(f"{base}_model.svg", size=("120mm", "120mm"))

dwg.viewbox(vb_minx, vb_miny, vb_width, vb_height)

for path in paths_um:

c2 = np.asarray(path, float)[:, :2]

dwg.add(dwg.polyline(c2.tolist(), stroke="black", fill="none", stroke_width=stroke_width_um))

cx, cy = (xmin + xmax) / 2.0, ymax + 0.25 * H

fs = max(7.0, 0.042 * max(W, H))

dwg.add(dwg.text(annotation_text(kind, p, iteration), insert=(cx, cy),

text_anchor="middle", font_size=f"{fs}", font_family="Arial",

font_weight="bold", fill="black"))

dwg.save()

print(f"🖼️ SVG → {base}_model.svg"); return f"{base}_model.svg"

# ─────────────────────────────────────────────

# COMPENSATION LOGIC

# Tracks w and h independently for each geometry configuration.

# ─────────────────────────────────────────────

def compute_compensated_params(geom_name: str, current: dict, measured: dict) -> dict:

"""

Computes next-iteration design values from current design values and

measured printed values (both in µm). Only w and h are adjusted.

Factor = current_design_value / measured_value.

"""

comp = dict(current)

def factor(cur_val, meas_key):

meas = measured.get(meas_key)

if meas is None or meas <= 0:

return 1.0

return cur_val / meas

f_w = factor(current["w"], "w_measured")

f_h = factor(current["h"], "h_measured")

comp["w"] = round(current["w"] * f_w)

comp["h"] = round(current["h"] * f_h)

print(f"\n📐 Compensation for {geom_name} (w and h only):")

print(f"   w: {current['w']} → {comp['w']} µm  (factor {f_w:.4f})")

print(f"   h: {current['h']} → {comp['h']} µm  (factor {f_h:.4f})")

return comp

def converged(measured: dict) -> bool:

"""Returns True when measured dimensions are within COMP_TOLERANCE_UM of nominal targets."""

w_ok = True

h_ok = True

if "w_measured" in measured:

w_ok = abs(measured["w_measured"] - DEFAULT["w"]) <= COMP_TOLERANCE_UM

if "h_measured" in measured:

h_ok = abs(measured["h_measured"] - DEFAULT["h"]) <= COMP_TOLERANCE_UM

return w_ok and h_ok

def get_measurements_from_user(geom_name: str, iteration: int, current_p: SimpleNamespace) -> dict:

"""Asks for measured w and h for a specific geometry. Press Enter to skip."""

print("\n" + "=" * 60)

print(f"COMPENSATION for {geom_name} — Iteration {iteration}")

print(f"  Target w = {DEFAULT['w']} µm  (current design = {current_p.w} µm)")

print(f"  Target h = {DEFAULT['h']} µm  (current design = {current_p.h} µm)")

print("  *Note: If a device does not use 'h' (like Spiral), just press Enter to skip it.")

print("=" * 60)

measured = {}

for key, label in [

("w_measured", "Measured outer channel width/diameter (µm)"),

("h_measured", "Measured height / horizontal length   (µm)"),

]:

raw = input(f"  {label}: ").strip()

if raw:

try:

measured[key] = float(raw)

except ValueError:

print(f"  ⚠️  Could not parse '{raw}', skipping.")

return measured

# ─────────────────────────────────────────────

# BUILDERS

# ─────────────────────────────────────────────

def build_spiral(p, suffix="", iteration=0):

print(f"🔄 Spiral{f' (iter {iteration})' if iteration > 0 else ''}...")

th = np.linspace(0, 2 * np.pi * p.loops, SPIRAL_SAMPLES)

r = p.r0 + p.s * th / (2 * np.pi)

x, y = r * np.cos(th), r * np.sin(th)

path_um = np.column_stack([x, y, np.zeros_like(x)])

path_mm = um2mm(path_um)

outer = um2mm(p.w)

inner_d = um2mm(p.internal_d)

outer2, _ = outward_thicken_params(outer, 0.0, OUTWARD_THICKEN_MM)

mesh = sweep_hollow_tube_volume(path_mm, outer2, inner_d, segments=TUBE_SEGMENTS)

base = os.path.join(OUTDIR, f"spiral_outward15{suffix}")

stl = f"{base}.stl"

mesh.export(stl)

print(f"✅ Spiral STL → {os.path.abspath(stl)}")

dxf = export_dxf_multi("Spiral", [path_um], p, base, iteration)

svg = export_svg_local_multi("Spiral", [path_um], p, base, iteration)

return stl, dxf, svg

def build_tjunction(p, suffix="", iteration=0):

print(f"🔄 T-junction{f' (iter {iteration})' if iteration > 0 else ''}...")

main_um = np.array([[-p.h / 2, 0, 0], [p.h / 2, 0, 0]])

branch_um = np.array([[0, 0, 0], [0, p.v, 0]])

outer = um2mm(p.w)

inner_d = um2mm(p.internal_d)

outer2, _ = outward_thicken_params(outer, 0.0, OUTWARD_THICKEN_MM)

wall2 = max((outer2 - inner_d) / 2.0, 1e-6)

R = outer2 / 2.0

r = max(R - wall2, 1e-6)

outer_main = sweep_solid_tube(um2mm(main_um),   R, TUBE_SEGMENTS, cap_start=True, cap_end=True)

outer_br   = sweep_solid_tube(um2mm(branch_um), R, TUBE_SEGMENTS, cap_start=True, cap_end=True)

main_mm_ext   = extend_polyline_at_start(extend_polyline_at_end(um2mm(main_um),   HOLLOW_BOOL_EPS_MM), HOLLOW_BOOL_EPS_MM)

branch_mm_ext = extend_polyline_at_start(extend_polyline_at_end(um2mm(branch_um), HOLLOW_BOOL_EPS_MM), HOLLOW_BOOL_EPS_MM)

inner_main = sweep_solid_tube(main_mm_ext,   r, TUBE_SEGMENTS, cap_start=True, cap_end=True)

inner_br   = sweep_solid_tube(branch_mm_ext, r, TUBE_SEGMENTS, cap_start=True, cap_end=True)

base = os.path.join(OUTDIR, f"tjunction_outward15{suffix}")

stl = f"{base}.stl"

export_difference([outer_main, outer_br], [inner_main, inner_br], stl)

canonicalize_stl_inplace(stl)

dxf = export_dxf_multi("T-junction", [main_um, branch_um], p, base, iteration)

svg = export_svg_local_multi("T-junction", [main_um, branch_um], p, base, iteration)

return stl, dxf, svg

def build_yjunction(p, suffix="", iteration=0):

print(f"🔄 Y-junction{f' (iter {iteration})' if iteration > 0 else ''}...")

ang = math.radians(p.angle)

main_um = np.array([[-p.h / 2, 0, 0], [0, 0, 0]])

b1_um = np.array([[0, 0, 0], [p.v * math.cos(ang),  p.v * math.sin(ang), 0]])

b2_um = np.array([[0, 0, 0], [p.v * math.cos(ang), -p.v * math.sin(ang), 0]])

outer = um2mm(p.w)

inner_d = um2mm(p.internal_d)

outer2, _ = outward_thicken_params(outer, 0.0, OUTWARD_THICKEN_MM)

wall2 = max((outer2 - inner_d) / 2.0, 1e-6)

R = outer2 / 2.0

r = max(R - wall2, 1e-6)

outer_main = sweep_solid_tube(um2mm(main_um), R, TUBE_SEGMENTS, cap_start=True, cap_end=True)

outer_b1   = sweep_solid_tube(um2mm(b1_um),   R, TUBE_SEGMENTS, cap_start=True, cap_end=True)

outer_b2   = sweep_solid_tube(um2mm(b2_um),   R, TUBE_SEGMENTS, cap_start=True, cap_end=True)

main_mm_ext = extend_polyline_at_start(extend_polyline_at_end(um2mm(main_um), HOLLOW_BOOL_EPS_MM), HOLLOW_BOOL_EPS_MM)

b1_mm_ext   = extend_polyline_at_start(extend_polyline_at_end(um2mm(b1_um),   HOLLOW_BOOL_EPS_MM), HOLLOW_BOOL_EPS_MM)

b2_mm_ext   = extend_polyline_at_start(extend_polyline_at_end(um2mm(b2_um),   HOLLOW_BOOL_EPS_MM), HOLLOW_BOOL_EPS_MM)

inner_main = sweep_solid_tube(main_mm_ext, r, TUBE_SEGMENTS, cap_start=True, cap_end=True)

inner_b1   = sweep_solid_tube(b1_mm_ext,   r, TUBE_SEGMENTS, cap_start=True, cap_end=True)

inner_b2   = sweep_solid_tube(b2_mm_ext,   r, TUBE_SEGMENTS, cap_start=True, cap_end=True)

base = os.path.join(OUTDIR, f"yjunction_outward15{suffix}")

stl = f"{base}.stl"

export_difference([outer_main, outer_b1, outer_b2], [inner_main, inner_b1, inner_b2], stl)

canonicalize_stl_inplace(stl)

dxf = export_dxf_multi("Y-junction", [main_um, b1_um, b2_um], p, base, iteration)

svg = export_svg_local_multi("Y-junction", [main_um, b1_um, b2_um], p, base, iteration)

return stl, dxf, svg

def write_csv(rows):

with open(CSV_PATH, "a", newline="", encoding="utf-8") as f:

w = csv.DictWriter(f, fieldnames=rows[0].keys())

if f.tell() == 0:

w.writeheader()

for r in rows:

w.writerow(r)

def main():

print("=== Trio (Independent w & h Compensation Loop) ===")

session = str(uuid.uuid4())[:8]

# Geometries map tracking their specific variables individually

geom_configs = {

"Spiral":     {"dict": dict(DEFAULT), "build_fn": build_spiral,    "converged": False},

"T-junction": {"dict": dict(DEFAULT), "build_fn": build_tjunction, "converged": False},

"Y-junction": {"dict": dict(DEFAULT), "build_fn": build_yjunction, "converged": False}

}

# ── Iteration 0: nominal ──

print("\n▶ Generating NOMINAL geometries (iteration 0)...")

rows = []

for label, cfg in geom_configs.items():

p = SimpleNamespace(**cfg["dict"])

t0 = time.time()

stl, dxf, svg = cfg["build_fn"](p, suffix="", iteration=0)

dt = time.time() - t0

rows.append({

"session":  session,

"device":   label,

"t_s":      f"{dt:.3f}",

"stl":      stl,

"dxf":      dxf,

"svg":      svg,

"stl_md5":  canonical_md5(stl),

})

write_csv(rows)

# ── Isolated Multi-iteration loop per geometry ──

iteration = 0

while True:

print(f"\n▶ Print iteration {iteration} files, measure each device, then enter values below.")

print(f"   Leave blank or press Enter to skip or freeze tracking for a device.")

any_active = False

new_rows = []

for label, cfg in geom_configs.items():

if cfg["converged"]:

continue

any_active = True

p = SimpleNamespace(**cfg["dict"])

measured = get_measurements_from_user(label, iteration + 1, p)

if not measured:

print(f"ℹ️  Stopping compensation loop for {label}.")

cfg["converged"] = True

continue

if converged(measured):

print(f"✅ {label} Converged successfully within target parameters!")

cfg["converged"] = True

continue

# Perform separate parameter updates for this component configuration

cfg["dict"] = compute_compensated_params(label, cfg["dict"], measured)

if not any_active or all(cfg["converged"] for cfg in geom_configs.values()):

print("\nℹ️  All geometries completed adjustments — exiting loop.")

break

iteration += 1

print(f"\n▶ Generating COMPENSATED geometries (iteration {iteration})...")

for label, cfg in geom_configs.items():

p = SimpleNamespace(**cfg["dict"])

t0 = time.time()

tag = f" [COMP iter={iteration}]"

stl, dxf, svg = cfg["build_fn"](p, suffix=f"_comp_iter{iteration}", iteration=iteration)

dt = time.time() - t0

new_rows.append({

"session":  session,

"device":   label + tag,

"t_s":      f"{dt:.3f}",

"stl":      stl,

"dxf":      dxf,

"svg":      svg,

"stl_md5":  canonical_md5(stl),

})

print(f"    {label} target design specs: w = {p.w} µm, h = {p.h} µm")

if new_rows:

write_csv(new_rows)

print(f"\n✅ Done → {OUTDIR}")

print(f"📊 CSV → {CSV_PATH}")

print(f"🧠 Log → {PROMPT_LOG}")

if __name__ == "__main__":

main()

# S5. Master Prompt:

Write a single Python script for generating three microfluidic devices — a Spiral, a T-junction, and a Y-junction — for FDM printing with a 0.2 mm nozzle. Each device exports an STL for printing and a DXF plus SVG for verification. The script includes a multi-iteration compensation loop that tracks width (w) and height (h) independently per geometry until each converges within tolerance of its nominal target. Output only code, nothing else.

Use exactly these imports and no others: os, math, time, uuid, hashlib, tempfile, subprocess, csv, struct, numpy as np, trimesh, svgwrite, ezdxf, SimpleNamespace from types.

Use exactly these constants: OUTDIR = "fdm_outward_fixed300_output", CSV_PATH = os.path.join(OUTDIR, "device_runs.csv"), PROMPT_LOG = os.path.join(OUTDIR, "prompt_log.txt"), OPENSCAD_EXE = r"C:\Program Files\OpenSCAD\openscad.exe", OUTWARD_THICKEN_MM = 0.2, HOLLOW_BOOL_EPS_MM = 0.15, TUBE_SEGMENTS = 64, SPIRAL_SAMPLES = 1200, COMP_TOLERANCE_UM = 5.0, DEFAULT = {"w": 600, "internal_d": 400, "r0": 200, "s": 2100, "loops": 5, "h": 1900, "v": 1500, "angle": 40}. All DEFAULT values are micrometers. Create OUTDIR with os.makedirs(OUTDIR, exist_ok=True) at module level.

Include log_prompt(kind, text) appending to PROMPT_LOG with a 60-dash separator, timestamp, kind, and text. Define um2mm(x) as np.asarray(x, float) / 1000.0. Define safe_norm(v) returning v divided by its norm if norm > 1e-12 else v unchanged. Define md5_of_file(path) returning "N/A" if path missing else hex MD5 in 4096-byte chunks. Define canonical_md5(stl_path) calling md5_of_file.

Define canonicalize_stl_inplace(stl_path, decimals=6). Load with trimesh.load(force="mesh"), concatenate if Scene, return early if no faces. Round vertices float64 to decimals, index tris = v[f], build keys by rounding tris reshaped to (n,9), sort with np.lexsort(keys.T[::-1]), reorder tris. Compute normals by crossing edge vectors, normalize, clamp zeros to 1.0, cast float32. Write binary STL with 80-byte header b"CANONICAL_STL_MD5_STABLE" left-justified with spaces, face count as little-endian uint32, each face as normal plus three vertices as float32 triples plus uint16 zero.

Define parallel_transport(P). Allocate T, Nn, Bb as (N,3) zeros. Tangents: index 0 forward diff, last index backward diff, others central diff, all safe_norm. Init up = [0,0,1.], fallback to [1.,0,0] if abs(dot(up,T[0])) > 0.95. Nn[0] = safe_norm(cross(T[0], cross(up,T[0]))), Bb[0] = safe_norm(cross(T[0],Nn[0])). Propagate with Rodrigues: axis = cross(T[i-1],T[i]), s = norm(axis), c = clip(dot(T[i-1],T[i]),-1,1). If s < 1e-6 copy Nn[i-1] and recompute Bb. Else v = axis/s, theta = atan2(s,c), K = skew(v), R = eye(3)+sin(theta)*K+(1-cos(theta))*(K@K), Nn[i] = safe_norm(R@Nn[i-1]), Bb[i] = safe_norm(cross(T[i],Nn[i])). Return T,Nn,Bb.

Define sweep_hollow_tube_volume(points_mm, outer_d_mm, inner_d_mm, segments=64). Raise if fewer than 2 points. R = max(outer/2,1e-6), r = max(inner/2,1e-6), if r>=R set r=0.9*R. Call parallel_transport. Build Vout and Vin ring arrays using circle = n[None,:]*ct[:,None]+b[None,:]*st[:,None] where ct,st from linspace(0,2*pi,segments,endpoint=False). Define ring_side_faces(off,rings,segs,flip=False) inside this function building pairs [[a,c,b],[b,c,d]], returning F[:,::-1] if flip. Fout = ring_side_faces(0,rings,segs,flip=False), Fin = ring_side_faces(len(Vout),rings,segs,flip=True). V = vstack([Vout,Vin]), F_list = [Fout,Fin]. Start annulus cap winding [[o0,o1,i0],[i0,o1,i1]] with o0=j, o1=(j+1)%segs, i0=len(Vout)+j, i1=len(Vout)+(j+1)%segs. End annulus cap winding [[o0,i0,o1],[i0,i1,o1]] with o0=(rings-1)*segs+j, i0=len(Vout)+(rings-1)*segs+j. Append both to F_list, vstack, create trimesh.Trimesh(process=True), fix_normals(), repair if not watertight. Return mesh.

Define extend_polyline_at_start(P, extension_mm): copy P float, P[0] += safe_norm(P[0]-P[1])*extension_mm, return P. Define extend_polyline_at_end(P, extension_mm): copy P float, P[-1] += safe_norm(P[-1]-P[-2])*extension_mm, return P.

Define sweep_solid_tube(points_mm, radius_mm, segments=64, cap_start=True, cap_end=True). Raise if fewer than 2 points. Clamp radius to 1e-6. Call parallel_transport. Build ring vertices, vstack to V. Define side_faces(rings_count,segs) inside using [[a,c,b],[b,c,d]]. Start cap: append P[0] to V, for each j add [[center_idx, b, a]] where a=j, b=(j+1)%segments. End cap: append P[-1] to V, for each j add [[center_idx, a, b]] where a=end0+j, b=end0+(j+1)%segments. Create trimesh.Trimesh(process=True), fix_normals(), raise if not watertight. Return mesh.

Define export_difference(outer_meshes, inner_meshes, outpath). Raise if OpenSCAD missing. Write outer{i}.stl and inner{i}.stl to tempfile.mkdtemp(prefix="scad_diff_") with forward-slash paths. Write diff.scad as difference(){ union(){ import each outer; } union(){ import each inner; } }. Run [OPENSCAD_EXE, "--render", "-o", outpath_abs, scad] capturing stdout and stderr. Raise RuntimeError with stderr if returncode != 0. Print success message.

Define outward_thicken_params(outer_d_mm, wall_mm, delta_mm) returning (outer_d_mm+2*delta_mm, wall_mm+delta_mm).

Define annotation_text(kind, p, iteration=0). Extra string: Spiral uses " | loops={p.loops} r0={p.r0}µm s={p.s}µm", T-junction uses " | h={p.h}µm v={p.v}µm", Y-junction uses " | h={p.h}µm v={p.v}µm θ={p.angle}°". Tag = "" if iteration==0 else f" [COMP iter={iteration}]". Return "{kind} | w={p.w}µm | internal_d={p.internal_d}µm | outer+0.2mm{extra}{tag}".

Define export_dxf_multi(kind, paths_um, p, base, iteration=0). outer2_um = (p.w/1000.0 + 2*OUTWARD_THICKEN_MM)*1000.0. Add lwpolyline per path XY with const_width=outer2_um/2.0 on layer "0". Bounding box of all points. cx=(xmin+xmax)/2, cy=ymax+0.25*H, text_height=max(35.0,0.056*H). Two overlapping text entities at (cx,cy) and (cx+0.02*text_height,cy) via try/set_placement with fallback to dxf.insert. Save as {base}.dxf, return path.

Define export_svg_local_multi(kind, paths_um, p, base, iteration=0). Bounding box, margin=0.45*max(W,H), viewbox at (xmin-margin,ymin-margin) size (W+2*margin,H+2*margin). stroke_width_um = (p.w/1000.0+2*OUTWARD_THICKEN_MM)*1000.0/3.0. svgwrite.Drawing size ("120mm","120mm"). Polylines stroke="black" fill="none". Text centered at cx=(xmin+xmax)/2, cy=ymax+0.25*H, font_size=max(7.0,0.042*max(W,H)), font_family="Arial", font_weight="bold", fill="black". Save as {base}_model.svg, return path.

Define compute_compensated_params(geom_name, current, measured). Both dicts in µm. Only w and h adjusted, all other keys copied unchanged. Inner factor(cur_val, meas_key) returns 1.0 if key missing or value <=0, else cur_val/measured[meas_key]. f_w = factor(current["w"],"w_measured"), f_h = factor(current["h"],"h_measured"). comp["w"] = round(current["w"]*f_w), comp["h"] = round(current["h"]*f_h). Print geom_name, both factors to 4 decimal places showing current to compensated, print all other parameters unchanged. Return comp.

Define converged(measured). w_ok=True, h_ok=True. If "w_measured" in measured: w_ok = abs(measured["w_measured"]-DEFAULT["w"]) <= COMP_TOLERANCE_UM. If "h_measured" in measured: h_ok = abs(measured["h_measured"]-DEFAULT["h"]) <= COMP_TOLERANCE_UM. Return w_ok and h_ok.

Define get_measurements_from_user(geom_name, iteration, current_p). Print header with geom_name, iteration number, target w and h from DEFAULT, current design w and h from current_p. Note that Spiral does not use h so user can press Enter to skip. Ask for w_measured and h_measured only. Parse as float, warn on failure. Return measured dict.

Define build_spiral(p, suffix="", iteration=0), build_tjunction(p, suffix="", iteration=0), build_yjunction(p, suffix="", iteration=0). All use the same geometry logic throughout every iteration. Spiral centerline: th=linspace(0,2*pi*p.loops,SPIRAL_SAMPLES), r=p.r0+p.s*th/(2*pi), path_um=column_stack([r*cos(th),r*sin(th),zeros]). T-junction: main_um=[[-h/2,0,0],[h/2,0,0]], branch_um=[[0,0,0],[0,v,0]]. Y-junction: main_um=[[-h/2,0,0],[0,0,0]], b1_um to [v*cos(ang),v*sin(ang),0], b2_um to [v*cos(ang),-v*sin(ang),0]. All R and r from: outer2 via outward_thicken_params(um2mm(p.w),0.0,OUTWARD_THICKEN_MM), wall2=max((outer2-um2mm(p.internal_d))/2.0,1e-6), R=outer2/2.0, r=max(R-wall2,1e-6). Spiral: sweep_hollow_tube_volume, export directly, no canonicalize. T and Y: sweep_solid_tube for outer and inner, extend each inner segment at both ends using extend_polyline_at_start(extend_polyline_at_end(seg,HOLLOW_BOOL_EPS_MM),HOLLOW_BOOL_EPS_MM), export_difference then canonicalize_stl_inplace. File bases: spiral_outward15{suffix}, tjunction_outward15{suffix}, yjunction_outward15{suffix}. All export calls pass iteration. Each builder returns (stl, dxf, svg).

Define write_csv(rows). Append mode, DictWriter with fieldnames from rows[0].keys(), write header only if file position is 0, write all rows.

Define main(). Print "=== Trio (Independent w & h Compensation Loop) ===". session = str(uuid.uuid4())[:8]. Build geom_configs dict with keys "Spiral", "T-junction", "Y-junction" each holding {"dict": dict(DEFAULT), "build_fn": the matching builder, "converged": False}. Generate nominal iteration 0 by looping over geom_configs, building each with suffix="" iteration=0, timing each, appending a row with keys session, device (label), t_s to 3 decimal places, stl, dxf, svg, stl_md5, calling write_csv on all rows together. Then enter while True loop. Print instruction to print and measure. Set any_active=False and new_rows=[]. Loop over geom_configs: skip if cfg["converged"]. Set any_active=True. Call get_measurements_from_user(label, iteration+1, SimpleNamespace(**cfg["dict"])). If no measurements print stopping message, set cfg["converged"]=True, continue. If converged(measured) print convergence message with measured and target values for w and h, set cfg["converged"]=True, continue. Otherwise call compute_compensated_params(label, cfg["dict"], measured) and store result back into cfg["dict"]. After the per-geometry loop: if not any_active or all converged, print all done message and break. Increment iteration. Print generating compensated geometry for iteration N. Loop over geom_configs again: build each with suffix=f"_comp_iter{iteration}" and iteration=iteration, time it, append row with device = label + f" [COMP iter={iteration}]", call write_csv on new_rows if any. Print each geometry's current w and h against nominal. After while loop print done messages for OUTDIR, CSV_PATH, PROMPT_LOG.